SIMATS ENGINEERING

ASSESSMENT TOOL 2 — ANSWER KEY

Course Name: Computer Networks   |   Course Code: CSA07   |   CO2 — IP Addressing, Subnetting & Basic Routing (BL3)

Name: pattan shoib khan|   Register No: 192524299  |   Course Code: 0703

# Question 1 — Smart Campus VLSM Subnetting

Given IP block: 192.168.100.0/24 (256 addresses total). Subnets required: School of Computing (/25), School of Electronics (/26), School of Mechanical Engineering (/27). Allocation is done using Variable Length Subnet Masking (VLSM), assigning the largest requirement first so that address blocks align correctly.

## 1 & 2. Network Address, Broadcast Address and Valid Host Range

| Department | Subnet Mask | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- | --- |
| School of Computing | /25 (255.255.255.128) | 192.168.100.0 | 192.168.100.127 | 192.168.100.1 – 192.168.100.126 |
| School of Electronics | /26 (255.255.255.192) | 192.168.100.128 | 192.168.100.191 | 192.168.100.129 – 192.168.100.190 |
| School of Mechanical Engg. | /27 (255.255.255.224) | 192.168.100.192 | 192.168.100.223 | 192.168.100.193 – 192.168.100.222 |

## 3. Usable Host Addresses per Subnet

| Department | Formula | Usable Hosts |
| --- | --- | --- |
| School of Computing (/25) | 2^7 − 2 | 126 |
| School of Electronics (/26) | 2^6 − 2 | 62 |
| School of Mechanical Engg. (/27) | 2^5 − 2 | 30 |

## 4. Remaining Unused IP Address Range

Total addresses allocated so far = 128 (/25) + 64 (/26) + 32 (/27) = 224 out of 256 addresses in 192.168.100.0/24.

Remaining unused block: 192.168.100.224 – 192.168.100.255 (a full /27 block of 32 addresses, network address 192.168.100.224, broadcast 192.168.100.255), reserved for future expansion.

# Question 2 — Multinational Company VLSM Subnetting

Given IP block: 10.10.0.0/16 (65,536 addresses total). Subnets required: Software Development (/24), Quality Assurance (/25), Human Resources (/26), Executive Management (/27). Allocation again follows VLSM, largest block first.

## 1 & 2. Network Address, Broadcast Address and Valid Host Range

| Department | Subnet Mask | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- | --- |
| Software Development | /24 (255.255.255.0) | 10.10.0.0 | 10.10.0.255 | 10.10.0.1 – 10.10.0.254 |
| Quality Assurance | /25 (255.255.255.128) | 10.10.1.0 | 10.10.1.127 | 10.10.1.1 – 10.10.1.126 |
| Human Resources | /26 (255.255.255.192) | 10.10.1.128 | 10.10.1.191 | 10.10.1.129 – 10.10.1.190 |
| Executive Management | /27 (255.255.255.224) | 10.10.1.192 | 10.10.1.223 | 10.10.1.193 – 10.10.1.222 |

## 3. Usable Hosts per Subnet

| Department | Formula | Usable Hosts |
| --- | --- | --- |
| Software Development (/24) | 2^8 − 2 | 254 |
| Quality Assurance (/25) | 2^7 − 2 | 126 |
| Human Resources (/26) | 2^6 − 2 | 62 |
| Executive Management (/27) | 2^5 − 2 | 30 |

## 4. Remaining Unused IP Address Range

Addresses consumed: 256 (/24) + 128 (/25) + 64 (/26) + 32 (/27) = 480 addresses, all within 10.10.0.0 – 10.10.1.223.

Remaining unused block: 10.10.1.224 – 10.10.255.255, which spans the rest of the allocated 10.10.0.0/16 block and remains available for future departments or expansion.

# Question 3 — Hospital IPv4-to-IPv6 Migration

Given IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

## 1. Compressed Notation

2001:db8:abcd::/64

Leading zeros in each hextet are dropped (0db8 → db8), and the longest run of consecutive all-zero hextets is replaced once with '::' (0000:0000:0000:0000:0000 → ::).

## 2. Expanded (Full Hexadecimal) Notation

2001:0db8:abcd:0000:0000:0000:0000:0000

Every hextet is written back to its full 4-hexadecimal-digit form, and '::' is expanded into the correct number of zero hextets so that 8 groups of 16 bits are shown.

## 3. Network Prefix (using /64)

Network Prefix = 2001:0db8:abcd:0000  (i.e. 2001:db8:abcd::/64)

The /64 prefix means the first 64 bits (the first four hextets) identify the network, while the remaining 64 bits form the Interface ID used for hosts/devices.

## 4 & 5. Total Number of Host Addresses Supported

Host portion = 128 − 64 = 64 bits.

Total possible addresses in the subnet = 2^64 = 18,446,744,073,709,551,616 addresses.

This enormous address space (over 18 quintillion addresses) is more than sufficient to uniquely address every medical device, IoT sensor, and connected equipment in the hospital network, even accounting for large future growth — this is the standard reason /64 is used for LAN subnets in IPv6.

# Question 4 — Longest Prefix Match Routing

Routing table entries: 192.168.1.0/24, 192.168.2.0/24, 192.168.3.0/24. Incoming packet destination: 192.168.2.45

## 1. Destination Subnet

192.168.2.0/24 — because 192.168.2.45 lies within the range 192.168.2.1 – 192.168.2.254.

## 2. Matching Routing Table Entry

192.168.2.0/24 is the matching entry (exact and only match — this is the Longest Prefix Match, since all three entries are /24 but only this one contains the destination address).

## 3. Next-Hop Network / Interface

The router forwards the packet out of the interface configured for the 192.168.2.0/24 network (i.e., the interface directly connected to, or the next-hop pointing toward, the 192.168.2.0/24 LAN).

## 4. Default Route (No Match Case)

If the destination address does not match any entry in the routing table, the router forwards the packet using the default route, denoted 0.0.0.0/0, which points to the gateway of last resort (typically the ISP's upstream router).

# Question 5 — Two-LAN Layer-3 Router Design

Administration LAN: 192.168.10.0/26   |   Finance LAN: 192.168.10.64/26

## 1. Network Address and Broadcast Address

| LAN | Network Address | Broadcast Address |
| --- | --- | --- |
| Administration LAN | 192.168.10.0 | 192.168.10.63 |
| Finance LAN | 192.168.10.64 | 192.168.10.127 |

## 2. Valid Host IP Address Range

| LAN | Valid Host Range | Usable Hosts |
| --- | --- | --- |
| Administration LAN | 192.168.10.1 – 192.168.10.62 | 62 (2^6 − 2) |
| Finance LAN | 192.168.10.65 – 192.168.10.126 | 62 (2^6 − 2) |

## 3. Suggested IP Addresses for Three Computers per LAN

| LAN | PC-1 | PC-2 | PC-3 |
| --- | --- | --- | --- |
| Administration LAN | 192.168.10.10 | 192.168.10.20 | 192.168.10.30 |
| Finance LAN | 192.168.10.70 | 192.168.10.80 | 192.168.10.90 |

(Any addresses within each LAN's valid host range, excluding the gateway address, are acceptable — the values above are illustrative.)

## 4. Default Gateway for Each LAN

| LAN | Default Gateway |
| --- | --- |
| Administration LAN | 192.168.10.1 |
| Finance LAN | 192.168.10.65 |

Hosts in the Administration LAN use 192.168.10.1 as their default gateway, and hosts in the Finance LAN use 192.168.10.65, as configured on the router's Layer-3 interfaces. This allows the router to route traffic between the two LANs and out to any external network.

— End of Answer Key —