SIMATS ENGINEERING

ASSESSMENT TOOL 3

| Field | Details |
| --- | --- |
| Student Name | Pattan shoib khan |
| Register Number | 192524299 |
| Course Code | 0703 |
| Course Name | Computer Networks (CSA07) |
| Course Outcome | CO2 – Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology (BL3) |
| Assessment | Annexure A – Debugging & Optimisation Task |
| Software | Cisco Packet Tracer, Wireshark |
| Weightage | 30% |

# Question 1 – IPv4 Subnet Configuration Debugging

## Given

- IP Address: 192.168.10.65

- Subnet Mask: 255.255.255.192 (/26)

- Default Gateway: 192.168.10.1

- Expected subnet: 192.168.10.0/26

## Step 1 – Identify the Subnet

A /26 mask (255.255.255.192) gives a block size of 256 − 192 = 64. The subnet boundaries in the 192.168.10.0/24 range are:

| Subnet | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- |
| 1 | 192.168.10.0 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 |
| 2 | 192.168.10.64 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 |
| 3 | 192.168.10.128 | 192.168.10.191 | 192.168.10.129 – 192.168.10.190 |
| 4 | 192.168.10.192 | 192.168.10.255 | 192.168.10.193 – 192.168.10.254 |

The address 192.168.10.65 falls in the range 64–127, so the workstation actually belongs to subnet 192.168.10.64/26, not 192.168.10.0/26.

## Step 2 – Network Address, Broadcast Address, Valid Host Range

- Network Address: 192.168.10.64

- Broadcast Address: 192.168.10.127

- Valid Host Range: 192.168.10.65 – 192.168.10.126

## Step 3 – Verify the IP and Gateway

- Configured IP (192.168.10.65) is a valid host address, but only inside subnet 192.168.10.64/26 — not inside the intended subnet 192.168.10.0/26.

- Default Gateway 192.168.10.1 belongs to subnet 192.168.10.0/26 (valid range .1–.62), which is a different subnet from the one the workstation is actually on.

- Because the host and its gateway sit in two different /26 subnets, the workstation cannot reach the gateway directly, and Layer 3 communication fails.

## Step 4 – Configuration Errors Identified

- Subnet mismatch: the workstation's IP places it in 192.168.10.64/26 while the rest of the LAN is on 192.168.10.0/26.

- Unreachable gateway: 192.168.10.1 is outside the workstation's own subnet range, so ARP/Layer-2 resolution to the gateway fails.

## Step 5 – Optimized Configuration

- New IP Address: 192.168.10.10  (any address between .1 and .62)

- Subnet Mask: 255.255.255.192 (/26) — unchanged

- Default Gateway: 192.168.10.1 — now valid, since it lies within the same subnet (192.168.10.0/26)

## Step 6 – Usable Hosts After Correction

Usable hosts = 2^(host bits) − 2 = 2^6 − 2 = 62 usable host addresses in 192.168.10.0/26.

# Question 2 – IPv6 Addressing Debugging

## Given

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

## Step 1 – Expand Both Addresses to Full Notation

| Device | Full (Expanded) IPv6 Address |
| --- | --- |
| Device A | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Device B | 2001:0db8:0000:0000:0001:0000:0000:0001 |

## Step 2 – Determine the Network Prefix of Each Device (/64)

The /64 prefix is formed from the first four hextets (first 64 bits):

- Device A prefix: 2001:0db8:0000:0000  →  2001:db8::/64

- Device B prefix: 2001:0db8:0000:0000  →  2001:db8::/64

## Step 3 – Verify Whether Both Devices Belong to the Same Subnet

Both devices resolve to the identical /64 network prefix 2001:db8::/64. Despite looking different in compressed ("::") notation, once fully expanded both addresses share the same network portion. So the two devices DO belong to the same subnet — there is no prefix mismatch.

## Step 4 – Identifying the Real Cause of the Failure

Since the addressing/prefix itself is correct, the communication failure must come from a non-addressing cause. Likely candidates include:

- A Duplicate Address Detection (DAD) conflict on the link.

- Failed Neighbor Discovery Protocol (NDP) resolution between the two interface IDs.

- A Layer-2 (switch/VLAN) connectivity problem preventing the devices from reaching each other despite being on the same logical subnet.

- A firewall or ACL blocking ICMPv6 (which NDP depends on).

## Step 5 – Optimized / Confirmed IPv6 Configuration

Both devices are kept on the same prefix, with clean, unambiguous interface IDs to remove any risk of confusion during troubleshooting:

- Device A: 2001:db8::1/64

- Device B: 2001:db8::2/64

Recommended debugging steps: verify Layer-2 link status, clear and rebuild the neighbor cache, and confirm no ACL/firewall is dropping ICMPv6 Neighbor Solicitation/Advertisement messages.

## Step 6 – Available Host Addresses in the Subnet

A /64 IPv6 subnet has 64 host (interface ID) bits, giving 2^64 (≈ 1.8 × 10^19) possible addresses. Unlike IPv4, IPv6 has no network/broadcast address to subtract, so effectively all interface IDs in the /64 are usable for hosts.

# Question 3 – VLSM Optimization of 192.168.1.0/24

## Step 1 – Subnet Details for Each Mask Used by the Organization

/26 subnets (block size 64, 4 subnets, 62 usable hosts each):

| Subnet | Network Address | Broadcast Address | Host Range | Usable Hosts |
| --- | --- | --- | --- | --- |
| 192.168.1.0/26 | 192.168.1.0 | 192.168.1.63 | .1 – .62 | 62 |
| 192.168.1.64/26 | 192.168.1.64 | 192.168.1.127 | .65 – .126 | 62 |
| 192.168.1.128/26 | 192.168.1.128 | 192.168.1.191 | .129 – .190 | 62 |
| 192.168.1.192/26 | 192.168.1.192 | 192.168.1.255 | .193 – .254 | 62 |

/27 subnets (block size 32, 8 subnets, 30 usable hosts each):

| Subnet | Network Address | Broadcast Address | Host Range | Usable Hosts |
| --- | --- | --- | --- | --- |
| 192.168.1.0/27 | .0 | .31 | .1 – .30 | 30 |
| 192.168.1.32/27 | .32 | .63 | .33 – .62 | 30 |
| 192.168.1.64/27 | .64 | .95 | .65 – .94 | 30 |
| 192.168.1.96/27 | .96 | .127 | .97 – .126 | 30 |
| 192.168.1.128/27 | .128 | .159 | .129 – .158 | 30 |
| 192.168.1.160/27 | .160 | .191 | .161 – .190 | 30 |
| 192.168.1.192/27 | .192 | .223 | .193 – .222 | 30 |
| 192.168.1.224/27 | .224 | .255 | .225 – .254 | 30 |

/28 subnets (block size 16, 16 subnets, 14 usable hosts each) — network address increases in steps of 16 from .0 to .240, each giving 14 usable host addresses.

## Step 2 – Usable vs Unused Addresses

- /26 → 64 total addresses, 62 usable, 2 reserved (network + broadcast) per subnet

- /27 → 32 total addresses, 30 usable, 2 reserved per subnet

- /28 → 16 total addresses, 14 usable, 2 reserved per subnet

Departments using a /26 while needing far fewer hosts leave large blocks of unused addresses, which is the core inefficiency reported.

## Step 3 – Example of an Inefficient (Fixed-Length) Allocation

Illustrative department requirements and the mistake in the original fixed-subnetting plan:

| Department | Hosts Needed | Assigned Subnet | Usable Hosts | Wasted Addresses |
| --- | --- | --- | --- | --- |
| Sales | 55 | 192.168.1.0/26 | 62 | 7 |
| HR | 25 | 192.168.1.64/26 | 62 | 37 |
| IT | 12 | 192.168.1.128/27 | 30 | 18 |
| Admin | 6 | 192.168.1.160/27 | 30 | 24 |
| Security (WAN link) | 2 | 192.168.1.192/28 | 14 | 12 |

This confirms the reported symptom: HR, Admin, and the Security link are each over-allocated, wasting a combined 98 addresses that other departments could have used.

## Step 4 – Debugging the Addressing Plan

- Root cause: subnet sizes were chosen from a fixed, limited set (/26, /27, /28 only) rather than being matched exactly to each department's real host count.

- Small point-to-point/low-host segments (e.g., a 2-host WAN link) were given /28 or /27 blocks instead of a much smaller block.

- No contiguous address block was reserved for future growth, since subnets were scattered inefficiently.

## Step 5 – Recommended VLSM-Optimized Scheme

Re-subnetting from largest host requirement to smallest, using the exact mask each department needs:

| Department | Hosts Needed | Optimized Subnet | Usable Hosts | Host Range |
| --- | --- | --- | --- | --- |
| Sales | 55 | 192.168.1.0/26 | 62 | .1 – .62 |
| HR | 25 | 192.168.1.64/27 | 30 | .65 – .94 |
| IT | 12 | 192.168.1.96/28 | 14 | .97 – .110 |
| Admin | 6 | 192.168.1.112/29 | 6 | .113 – .118 |
| Security (WAN link) | 2 | 192.168.1.120/30 | 2 | .121 – .122 |

This VLSM plan reduces wastage to a minimum per subnet and leaves 192.168.1.124 – 192.168.1.255 (132 addresses) free as a contiguous reserved block for future departments or expansion — a clear improvement over the original scattered fixed-length allocation.

# Question 4 – IPv6 Addressing Debugging (Repeat of Question 2)

Question 4 uses the same two addresses as Question 2, so the debugging steps and conclusion are identical.

## Summary of Findings

| Step | Result |
| --- | --- |
| Device A expanded | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Device B expanded | 2001:0db8:0000:0000:0001:0000:0000:0001 |
| Device A /64 prefix | 2001:db8::/64 |
| Device B /64 prefix | 2001:db8::/64 |
| Same subnet? | Yes — both share the identical /64 prefix |
| Actual issue | Not addressing; likely NDP / DAD / L2 connectivity / ACL blocking ICMPv6 |
| Optimized addresses | Device A: 2001:db8::1/64, Device B: 2001:db8::2/64 |
| Available hosts in /64 | 2^64 (≈1.8 × 10^19) — effectively unrestricted |

# Question 5 – Routing Table Debugging

## Given Routing Table

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route (0.0.0.0/0) | 10.0.0.1 |

Reported problem: packets destined for 192.168.2.100 are not reaching the intended network.

## Step 1 – Longest-Prefix Match for 192.168.2.100

- Against 192.168.1.0/24 → no match (different third octet).

- Against 192.168.2.0/24 → match, /24 prefix.

- Against the default route 0.0.0.0/0 → also technically matches everything, but has the shortest prefix (/0).

By the longest-prefix-match rule, the router selects 192.168.2.0/24, not the default route.

## Step 2 – Next Hop Selected

Next hop for 192.168.2.100 is 10.0.0.3, as specified against the 192.168.2.0/24 entry.

## Step 3 – Verify the Destination Belongs to the Intended Subnet

192.168.2.0/24 has host range 192.168.2.1 – 192.168.2.254. The address 192.168.2.100 falls correctly inside this range, confirming the match is valid.

## Step 4 – Routing Configuration Errors

The routing table entry itself is correct and complete — 192.168.2.0/24 is present and correctly points to 10.0.0.3. There is no missing route or typo in the table shown. Since packets still fail to arrive, the fault lies outside the table, most likely one of:

- The next-hop router at 10.0.0.3 is down or its interface is administratively/physically disabled.

- 10.0.0.3 has no return route back to the source network, causing asymmetric routing and dropped replies.

- A firewall/ACL along the path is blocking traffic to 192.168.2.0/24.

- A duplicate or conflicting static route elsewhere in the network overriding this path.

## Step 5 – Optimizing the Routing Table

- Verify reachability of 10.0.0.3 (ping/traceroute) and confirm the interface facing it is up.

- Confirm 10.0.0.3 has a route back to the source network to avoid asymmetric routing.

- Where possible, summarize/aggregate contiguous networks to reduce table size and lookup overhead.

- Enable route monitoring (e.g., keepalives) so a failed next hop is detected and rerouted automatically.

## Step 6 – Route Used if No Matching Destination Exists

If a packet's destination does not match either 192.168.1.0/24 or 192.168.2.0/24, the router falls back to the Default Route and forwards it to 10.0.0.1.