SIMATS ENGINEERING

ASSESSMENT TOOL 3 — MOCK INTERVIEW ANSWERS

| Name | Pattan shoib khan |
| --- | --- |
| Register No | 192524299 |
| Course Code | 0703 |
| Course Name | Computer Networks |
| Course Outcome | CO5: Measure network performance using key metrics with simulation tools |

# Section A: Analytical & Evaluative Questions

Q1. Analyze how network congestion affects packet delay and packet loss. (BL4)

Answer: When congestion occurs, more packets arrive at a router than it can forward, so packets queue up in buffers. This queuing increases delay because each packet waits longer before transmission. Once buffers overflow, incoming packets are dropped, causing packet loss. Both effects reduce throughput and force retransmissions (in TCP), which further worsens congestion.

Q2. Compare the impact of star and mesh topologies on network reliability and fault tolerance. (BL4)

Answer: In a star topology, all devices connect to a central hub or switch; if that central device fails, the entire network goes down, making it less fault-tolerant. In a mesh topology, devices have multiple interconnections, so if one link or node fails, traffic can be rerouted through alternate paths. Mesh offers higher reliability and fault tolerance but at a higher cost and complexity, while star is simpler and cheaper but has a single point of failure.

Q3. Why does increasing the number of routers between source and destination increase end-to-end delay? Explain logically. (BL4)

Answer: Each router a packet passes through adds processing delay (examining the header), queuing delay (waiting in the buffer), and transmission delay (sending it onto the next link). As the number of hops increases, these per-hop delays accumulate, increasing total end-to-end delay even if each individual router is fast.

Q4. Analyze how Quality of Service (QoS) improves the performance of voice and video traffic. (BL4)

Answer: QoS mechanisms such as traffic classification, prioritization, and bandwidth reservation ensure that delay-sensitive traffic like voice and video is forwarded ahead of less sensitive data. By reducing delay, jitter, and packet loss for these flows, QoS maintains smooth, real-time communication even when the network is under load.

Q5. Evaluate the impact of using fiber optic cables instead of copper cables for high-speed communication. (BL5)

Answer: Fiber optic cables offer much higher bandwidth, lower attenuation, and immunity to electromagnetic interference compared to copper. This allows data to travel longer distances at higher speeds with fewer errors and repeaters, making fiber far more suitable for high-speed, long-distance communication, though it is costlier to install and requires specialized equipment.

Q6. Analyze how network segmentation using VLANs improves performance and security. (BL4)

Answer: VLANs divide a physical network into logical broadcast domains, so broadcast traffic from one group of devices does not flood the entire network, improving performance. They also isolate traffic between departments or user groups, so sensitive systems are not directly reachable from other segments, which improves security by limiting the attack surface.

Q7. Compare the effects of TCP and UDP on network performance for different applications. (BL4)

Answer: TCP is connection-oriented and guarantees reliable, ordered delivery through acknowledgments and retransmissions, which adds overhead and delay — suitable for file transfer and web browsing. UDP is connectionless with no guarantee of delivery or order, giving lower overhead and delay, which suits real-time applications like video streaming and VoIP where speed matters more than occasional packet loss.

Q8. Why is load balancing important in large enterprise networks? Analyze its impact on performance and availability. (BL5)

Answer: Load balancing distributes traffic across multiple servers or links so no single resource is overwhelmed. This prevents bottlenecks, reduces response time, and improves overall throughput. It also improves availability, because if one server fails, traffic can be redirected to healthy servers, avoiding downtime.

Q9. Analyze how frequent routing updates influence network efficiency and bandwidth utilization. (BL4)

Answer: Frequent routing updates help the network adapt quickly to topology changes, improving path accuracy. However, they consume additional bandwidth and processing resources on routers, since update packets are exchanged often. If updates are too frequent, this overhead can reduce the bandwidth available for actual data traffic, so there is a trade-off between responsiveness and efficiency.

Q10. Evaluate the trade-off between network security measures (such as firewalls and encryption) and overall network performance. (BL5)

Answer: Firewalls and encryption inspect and transform traffic, which adds processing delay and can reduce throughput, especially under heavy load. Stronger security typically means more computation per packet. Network designers must balance the level of security needed against acceptable performance, often using dedicated hardware (like ASIC-based firewalls) to minimize this trade-off.

# Section B: Problem-Solving / Troubleshooting Questions

Q11. A network has high delay—how will you identify and fix the issue? (BL4)

Answer: I would use tools like ping and traceroute to measure delay at each hop and identify where the delay is introduced. Likely causes include congestion, an overloaded router, a long physical path, or insufficient bandwidth. Based on the cause, I would fix it by upgrading bandwidth, optimizing routing, adding QoS, or load balancing traffic.

Q12. If throughput is low despite high bandwidth, what could be the reasons? (BL4)

Answer: Possible reasons include network congestion, high packet loss causing retransmissions, protocol overhead, an undersized TCP window, hardware limitations at the source or destination, or interference in wireless links. High bandwidth is only the capacity; actual throughput depends on how efficiently that capacity is used.

Q13. How would you reduce packet loss in a congested network? (BL4)

Answer: I would increase buffer sizes appropriately, implement QoS to prioritize critical traffic, use congestion control algorithms, upgrade bandwidth on congested links, and apply traffic shaping or load balancing to distribute traffic more evenly across available paths.

Q14. A VoIP call is breaking frequently—how would you troubleshoot? (BL4)

Answer: I would check for high jitter, delay, and packet loss on the path using diagnostic tools. I would verify QoS settings are prioritizing voice traffic, check for bandwidth congestion, and inspect the network for misconfigured devices or interference on wireless links, then apply fixes such as QoS policies or bandwidth upgrades.

Q15. Given a network with high jitter, suggest improvements. (BL4)

Answer: I would implement QoS with traffic prioritization for real-time flows, use jitter buffers at the receiving end, ensure consistent routing paths to avoid variable delay, and reduce congestion by upgrading bandwidth or balancing load across links.

Q16. How will you simulate a congested network in Packet Tracer? (BL5)

Answer: I would build a topology with limited bandwidth links, connect multiple end devices generating heavy traffic (e.g., using ping floods or simulated data transfers) toward a shared router or link, and then use Packet Tracer's simulation mode to observe increased delay, queuing, and dropped packets on the bottleneck link.

Q17. If packets are delayed in a router, what steps will you take? (BL4)

Answer: I would check the router's CPU and memory utilization, examine queue lengths and buffer occupancy, verify routing table efficiency, and check for misconfigured QoS policies. Depending on the cause, I would upgrade hardware, optimize configuration, or apply traffic prioritization to reduce delay.

Q18. How would you optimize a network for online gaming? (BL4)

Answer: I would prioritize gaming traffic using QoS to minimize delay and jitter, use a wired connection instead of Wi-Fi where possible, choose low-latency routing paths, and ensure sufficient bandwidth is reserved so gaming packets are not competing with bulk downloads.

Q19. Design a small network and explain how you will measure its performance. (BL5)

Answer: I would design a small LAN with a router, a switch, and a few end devices connected in a star topology. To measure performance, I would use ping to measure delay and packet loss, use file transfers to measure throughput, and use Packet Tracer's simulation mode to observe queuing and jitter under varying traffic loads.

Q20. If RTT is very high, how will you diagnose the issue? (BL4)

Answer: I would use traceroute to identify which hop contributes most to the round-trip time, check for congestion or misrouting at that hop, verify the physical distance and link quality, and check whether the destination server itself is slow to respond.

# Section C: Presentation & Explanation Questions

Q21. Explain network performance to a non-technical person. (BL5)

Answer: Network performance is like how well traffic flows on a road. If the road is wide and clear, cars (data) move quickly and smoothly. If there are too many cars, traffic jams (congestion) form, cars slow down (delay), and some may even have to turn back (packet loss). Good network performance means data reaches its destination quickly and reliably, just like clear roads let cars reach their destination fast.

Q22. Describe throughput using a real-life example. (BL5)

Answer: Throughput is like the actual amount of water flowing out of a tap in a minute, even if the pipe is rated for a larger maximum flow. Similarly, a network link may have a maximum bandwidth of 100 Mbps, but due to congestion or overhead, the actual data delivered — the throughput — might only be 70 Mbps.

Q23. Present how packet loss affects video streaming. (BL5)

Answer: When packets carrying video data are lost, the video player either has to skip that portion, causing visible glitches or freezing, or wait for retransmission, causing buffering delays. High packet loss results in poor video quality, stuttering playback, and a frustrating viewing experience, even if overall bandwidth is sufficient.

Q24. Explain delay types clearly in under 1 minute. (BL4)

Answer: There are four main types of delay: processing delay, the time a router takes to examine a packet's header; queuing delay, the time a packet waits in a buffer before being forwarded; transmission delay, the time to push all the packet's bits onto the link; and propagation delay, the time for a signal to physically travel across the medium. Total end-to-end delay is the sum of these across all hops.

Q25. Describe Cisco Packet Tracer in a professional way. (BL5)

Answer: Cisco Packet Tracer is a network simulation tool that allows users to design, configure, and test network topologies in a virtual environment. It supports a wide range of devices such as routers, switches, and end hosts, and lets users simulate real-world protocols and traffic behavior, making it valuable for learning and validating network designs before real-world deployment.

Q26. Explain QoS in simple and technical terms. (BL5)

Answer: Simply put, QoS means giving important traffic priority so it doesn't get stuck behind less important traffic — like an ambulance being allowed to pass through traffic first. Technically, QoS refers to a set of mechanisms, such as traffic classification, marking, queuing, and bandwidth allocation, used to prioritize certain types of network traffic to meet performance requirements like low delay and low jitter.

Q27. How would you present your simulation results to a client? (BL4)

Answer: I would summarize the key performance metrics — delay, throughput, jitter, and packet loss — using clear charts and graphs rather than raw data. I would explain what each metric means for their business (e.g., how it affects call quality or download speed), highlight any issues found, and present practical recommendations to improve performance.

Q28. Explain jitter using a real-world analogy. (BL5)

Answer: Jitter is like a bus that is supposed to arrive every 5 minutes but sometimes arrives in 3 minutes and sometimes in 8 minutes. The variation in arrival time, rather than the average time itself, is jitter. In networks, jitter is the variation in packet arrival times, which can disrupt real-time applications like voice and video calls.

Q29. Describe how you tested network performance in a lab. (BL4)

Answer: In the lab, I built a small topology in Packet Tracer with routers, switches, and PCs. I used ping to measure delay and packet loss, generated file transfers to observe throughput, and used the simulation mode to watch how packets queued at routers under increased traffic load, noting how performance changed as I adjusted bandwidth and added congestion.

Q30. Explain the importance of performance metrics in one structured answer. (BL5)

Answer: Performance metrics such as delay, throughput, jitter, and packet loss are important because they provide measurable evidence of how well a network is functioning. They help identify bottlenecks, guide capacity planning, support troubleshooting, and ensure that service level agreements are met, ultimately ensuring a reliable experience for end users.

# Section D: Personal & Behavioral (Interview) Questions

Q31. Introduce yourself and explain your knowledge of network performance. (BL5)

Answer: I am a second-year Information Technology student studying Computer Networks as part of my curriculum. I have learned how to analyze key performance metrics such as delay, throughput, jitter, and packet loss, and I have practical experience simulating and measuring these metrics using Cisco Packet Tracer, which has strengthened my understanding of how real networks behave under different conditions.

Q32. Why are you interested in networking and simulation tools? (BL5)

Answer: I am interested in networking because it forms the backbone of nearly every modern technology, and understanding how data moves helps me build a strong foundation for further studies in IT and cybersecurity. Simulation tools like Packet Tracer let me experiment with network designs and observe real behavior without needing expensive physical hardware, which makes learning practical and engaging.

Q33. What makes you confident in analyzing network performance? (BL5)

Answer: My confidence comes from hands-on practice — I have designed topologies, generated traffic, and measured metrics like delay and throughput in simulation tools, which helped me connect theoretical concepts to observable results. This practical exposure, combined with consistent study of the underlying protocols, gives me confidence in analyzing and troubleshooting performance issues.

Q34. Explain a concept you are very strong in (related to networking). (BL5)

Answer: I am strong in understanding the different types of delay — processing, queuing, transmission, and propagation delay — and how they combine to affect end-to-end performance. I can clearly explain how each type arises and how specific network conditions, like congestion or long distances, influence each one.

Q35. How would you handle a question you don't know? (BL5)

Answer: I would stay calm and honestly acknowledge that I am not fully certain, rather than guessing incorrectly. I would then reason through the problem using related concepts I do understand, explain my logical approach, and mention that I would verify the exact answer through further study or research.

Q36. Describe a situation where you solved a technical problem. (BL4)

Answer: While working on a network simulation, I noticed a link was experiencing consistent packet loss. I traced the issue to an overloaded router in the topology and resolved it by redistributing traffic through an alternate path and enabling a basic QoS policy, which restored normal performance.

Q37. Why should we select you for a networking role? (BL5)

Answer: I have a solid theoretical understanding of networking concepts combined with practical simulation experience, I am comfortable analyzing performance metrics and troubleshooting issues logically, and I am eager to continue learning and applying new tools and technologies, which makes me a strong fit for a role that requires both technical skill and adaptability.

Q38. What challenges did you face while learning Packet Tracer? (BL5)

Answer: Initially, I found it challenging to correctly configure routing protocols and interpret simulation results. I overcame this by practicing repeatedly with small topologies, referring to documentation, and gradually building more complex networks, which helped me understand both the tool and the underlying networking concepts more deeply.

Q39. How do you improve your technical skills regularly? (BL5)

Answer: I regularly practice hands-on exercises in simulation tools, read updated networking resources and documentation, work on small projects to apply what I learn, and revisit challenging topics until I fully understand them, which helps me build both depth and confidence over time.

Q40. Demonstrate confidence while explaining a complex concept simply. (BL5)

Answer: Take Quality of Service, for example — it simply means the network decides which traffic gets to go first, similar to how an emergency vehicle gets priority on the road. Technically, it involves classifying, marking, and prioritizing traffic so critical applications like voice calls perform smoothly even when the network is busy. I am confident explaining this because I have both studied the theory and tested it in simulation.